Electrical documentation

The electrical documentation for the project includes the following documents:

According to the requirements of the specifications.

Technical data

Requirements for the installation location

According to the requirements of the specifications.

Electrical feed

Requirements for the electrical feed can be found in the electrical circuit diagram.

|  | NOTE |
| --- | --- |
|  | Technical data for the units of other suppliers can be found in the documentation of the respective suppliers. |

Safety

Qualified persons in compliance with the safety instructions may only carry out all work on the electrical system and after the system operator has instructed them.

These persons must be familiar with the nationally applicable laws and regulations as well as internal plant regulations on occupational safety and accident prevention.

All safety notes and instructions in this operating manual must be observed.

Intended use

The electrical system incl. the control cabinets and the electrical control are intended exclusively for the machine / system intended for this purpose. Any other use beyond this is considered improper.

Only the process parameters / technical data specified in the operating instructions / circuit diagram / parts list may be used.

If deviating process parameters / technical data are set or deviating components are used, it is recom-mended that this be requested from the machine manufacturer.

Improper use

The following is considered improper use:

- Operation in violation of the defined performance data

- Use of unqualified or insufficiently qualified personnel

- Failure to comply with the safety instructions

- Operation of the plant / machine without fully functional safety devices.

- Setting of safety parameters to non-safe values.

- Unauthorized additions or modifications to the control cabinet or the control system that may impair safety.

- Failure to observe the maintenance intervals.

- Manipulation of the machine/system

|  | DANGER |
| --- | --- |
|  | Improper use Improper use may lead to fatal injuries to persons located on or in the system as well as to considerable property damage. Only use the plant / line for its intended, designated purpose. |

Conformity with EU directives

The electrotechnical system is compliant with the EU Directive 2014/35/EU (Low Voltage Directive).

In addition to the Low Voltage Directive, these EU directives were included in the design and manufacture of the machine:

- EMC-Directive (2014/30/EU)

The following electrical engineering standards have been taken into account in the design, installation and commissioning of this machine:

- EN 60204-1:2018

Safety of machinery - Electrical equipment of machines - Part 1: General requirements

- EN IEC 61439-1:2021

Low-voltage switchgear and control gear assemblies - Part 1: General rules

- EN IEC 61439-2:2021

Low-voltage switchgear and control gear assemblies - Part 2: Power switchgear and control gear assemblies

Basic safety instructions

The basic prerequisite for the safe handling and error-free operation of the electrical system is the knowledge and observance of the basic safety instructions, the safety instructions in the individual chapters, as well as in the associated documents and in the supplier documents.

|  | DANGER |
| --- | --- |
|  | Electric voltage Authorized personnel may only carry out work on the electrical system when it is switched off and the mains disconnection device is secured. The five safety rules must be observed. These are assumed to be known by a qualified electrician: - Disconnect - Secure against re-connection - Verification of no voltage - Ground and short circuit - Provide protection against adjacent live parts |
|  | DANGER |
|  | Non-compliance with the safety instructions In order to avoid dangers, all specified safety instructions and handling instructions must be observed and complied with. All danger, warning and information signs must be attached to the system so that they are clearly visible and must not be removed or covered. |
|  | DANGER |
|  | Structural modifications No significant changes, additions or conversions may be made to the electrical system without the manufacturer's approval. |
|  | DANGER |
|  | Plant parameters Preset plant parameters must not be changed. Adjustable protective devices must be set according to the values as specified in the operating instructions/circuit diagram. |
|  | DANGER |
|  | Residual energy Some equipment (servocontrollers, frequency converters, ...) with an electrical DC link may still have dangerous residual voltages for a certain time after disconnection. Check that no voltage is present before starting work. |
|  | NOTE |
|  | The operating instructions must be stored close to the plant. The operating instructions must be readily available and freely accessible to the operating and maintenance personnel at all times. For completeness, the operating instructions of the sub-suppliers must be kept together with this operating instruction. The operating instructions of the sub-suppliers, in particular the safety instructions given there, must always be observed. |

Fire extinguishing equipment

In case of fire, hazardous or endangered parts of the power supply shall be disconnected unless they must be kept energized for firefighting purposes or unless other hazards result from the disconnection.

|  | DANGER |
| --- | --- |
|  | Risk of death or injury from electric shock Never use water to extinguish electrical equipment! |

Obligation of the operator

The operator of the machine is responsible for defining the areas of activity, duties and competencies. He must also ensure that only qualified personnel instructed by him work on the machine and its components.

The recommendation of work equipment and the safety instructions of this operating manual do not replace the prescribed risk analysis of the operator in accordance with EEC Directive 89/391/EEC on the "Implementation of measures to improve the safety and health protection of workers at work".

The operator is obliged to allow only persons to work on the electrical installation who are

- are familiar with the nationally applicable laws and regulations, as well as internal plant regulations on work safety and accident prevention.

- are suitable, trained and instructed to work independently on the electrical system.

- have read and understood these operating instructions.

Obligation of the personnel

Before starting work all persons, who are assigned to work on the electrical system, are obliged to:

- observe the basic regulations on work safety and accident prevention.

- read and observe the chapter on safety and the applicable safety notes and instructions in this operation manual.

Personnel selection

Only authorized persons may work on the system. Only personnel qualified for the tasks may work on the system.

A qualified electrician may only make all electrical connections to a properly installed power supply system in compliance with the applicable safety regulations for work on electrical system parts. The applicable safety regulations are assumed to be known by a qualified electrician.

A qualified electrician is a person who, based on his technical training, knowledge and experience as well as knowledge of the relevant regulations, is able to assess the work assigned to him and recognize possible dangers.

Skilled personnel with specialist knowledge and in compliance with the relevant safety regulations may only carry out all work on pneumatic and hydraulic equipment.

| Personnel | Qualification |
| --- | --- |
| Operating personnel | Persons who have been trained and informed by the plant operator about the special features of the machine and the hazards it presents. Operating personnel are the personnel who constantly work on the equipment and perform tasks for the normal production process. |
| Mechanics | Persons with a recognized completed training in the metal sector or with equivalent experience. These persons have been trained and informed by the plant operator about the operation of the machine and the hazards it presents. Mechanics are responsible, for example, for maintenance and repair work in the mechanical area. |
| Electricians | Persons who have a recognized, completed electrotechnical training or with equivalent experience. These persons have been trained and informed by the system operator about the operation of the machine and the hazards it presents. Qualified electricians carry out e.g. maintenance and repair work in the field of electrical engineering. |
| Programmers | Persons with a recognized completed training in programming or with equivalent experience. These persons have been instructed and informed by the system operator about the operation of the machine and the hazards it presents. Robot programmers must complete additional training for the robots used. Persons who change safety-relevant parameters must have completed a certified training course. |
| Service personnel | Persons of F.EE GmbH or persons commissioned by F.EE GmbH who have in-depth knowledge and experience with the products distributed by F.EE GmbH. |

Personnel protective equipment

For all work on the plant, personal protective equipment must be worn as prescribed by the operator, taking into account the relevant safety regulations.

|  | DANGER |
| --- | --- |
|  | Without personal protective equipment, the risk of injury increases. All persons who are assigned to work on the system are required to wear the prescribed protective equipment properly. The operating instructions of the machine manufacturer need to be observed as well. |

Electrical safety

General

- Only use qualified electricians for work on electrical equipment.

- All necessary warning signs must be clearly visible and must not be covered.

- Labeling, warning signs and reference signs of the electrical equipment according to EN 60204-1 (chap. 16).

- Maintenance and cleaning work must only be carried out with the mains disconnection device secured. Secure the disconnecting device in the OFF position to prevent it from being switched on again (e.g. by padlock; warning sign if disconnecting device is inside housing).

- For work on the electrical installation, observe the five safety rules:

- Disconnect

- Secure against re-connection

- Verification of no voltage

- Ground and short circuit

- Provide protection against adjacent live parts

- For maintenance work involving the disassembly and assembly of machine components, the mains disconnection device in the control cabinet must be used.

Excluded electrical circuits

Excluded circuits according to EN 60204-1 can be recognized as follows:

- In the vicinity of excluded electrical circuits a sign is placed:

AND:

- The conductors are identifiable by color, taking into account the recommendation in EN 60204-1

Residual voltage

|  | DANGER |
| --- | --- |
|  | Residual energy Some equipment (servocontrollers, frequency converters, ...) with an electrical DC link may still have dangerous residual voltages for a certain time after disconnection. Check that no voltage is present before starting work. |

Protective conductor / Equipotential bonding

|  | DANGER |
| --- | --- |
|  | Immediate danger for life and limb Only connect the protective conductor to the terminal provided for this purpose in the control cabinet. Check the conductivity of the protective conductor. |

Machine safety (Protective devices)

The position and effective range of the protective devices and the emergency stop buttons can be found in the machine manufacturer's documentation.

|  | DANGER |
| --- | --- |
|  | Risk of death! There is risk of death if the system is operated without protective devices or with protective devices that are not in working order! There is also a risk of death if the system is switched on when persons are present in the danger zone. Only trained and qualified personnel may carry out the setup and manual operation. Before carrying out a movement, the operator must be sure that he has the actuator to be operated in view and can intervene in an emergency. During all work in setup and manual mode, make sure that no other persons are in the danger zone of the system. Before acknowledging protective devices, the person acknowledging them must make sure that there are no persons in the danger zone. |

|  | NOTE |
| --- | --- |
|  | The safety instructions in the operating manual of the machine manufacturer must be observed. |

|  | NOTE |
| --- | --- |
|  | Safety-related components must be replaced after 20 years in accordance with EN ISO 13849-1, unless the associated calculations show a shorter service life! |

Transport

To avoid component damage and hazards during transport, the following safety notes and instructions must be observed and complied with:

- Qualified and authorized personnel must perform transport only.

- Only suitable load handling attachments with sufficient load-bearing capacity may be used for transport. The weight of the goods to be transported must be taken into account

- No components may be damaged when applying the load handling attachments.

- Only use the lugs or heavy-duty lugs provided.

- Observe the load on the transport base according to the manufacturer's instructions.

- When lifting, make sure that the load is evenly distributed and that the goods to be transported do not tip over. The transported goods must always be lifted at the center of gravity.

- Moveable assemblies must be secured against unexpected changes in position. Do not release any securing measures applied to moveable assemblies until the transported good has reached its final installation location.

- If sufficient visibility is not ensured during a transport maneuver, appropriately trained instruction personnel must be available who are able to monitor the transport and issue the necessary instructions by means of signs.

|  | DANGER |
| --- | --- |
|  | Suspended load A fracture of the lifting gear or unintentional start of a movement of the load poses a serious risk to persons in the danger zone. - Never stand under suspended loads! - Make sure that no persons step under the lifted load and keep a suitable distance around the lifted load. |
|  | DANGER |
|  | Forklifts Improper handling of forklifts can result in injuries to persons and damage to property. - Any forklifts may only be operated by authorized personnel with special care. - The operating personnel of the plant must pay attention to the forklifts. |
|  | DANGER |
|  | Stability When transporting the system components, hazards may arise due to overturning, falling and uncontrolled changes in position. If adequate stability cannot be guaranteed due to the shape or intended way of installation, suitable fasteners and supports must be provided. |

Assembly

The following general notes on the installation, assembly and operation of the control cabinet must be observed:

- Personnel qualified to do so may only carry out assembly work. (See section 3.5.4 Personnel selection) The work must be carried out conscientiously and with care. Personal protective equipment must be used.

- All information on the assembly, installation and connection of the electrical system must be observed.

- The necessary system layouts, installation plans, etc. must be available to the installation personnel.

- The foundation specifications and floor bearing capacity at the installation site of the system must be taken into account.

- All installation instructions for the individual system components must be observed.

- Unnecessary openings on the control cabinet must be avoided. Any opening that is not carefully made and sealed can lead to a reduction in the degree of protection.

- Fastening of the control cabinet system and its components or equipment parts according to the rules of technology with sufficiently mechanically over dimensioned fasteners.

- Secure detachable connections against unintentional loosening with spring washers, toothed lock washers, liquid-applied thread locks.

Electrical feed

Requirements for the electrical feed can be found in the electrical circuit diagram.

|  | DANGER |
| --- | --- |
|  | Electrical voltage A qualified electrician may only make all electrical connections to a properly installed power supply system in compliance with the applicable safety regulations for work on electrical system parts. |
|  | DANGER |
|  | Stability All plant components must be force-fitted to the building floor to avoid unexpected changes in position. All connections must be checked regularly for strength and material fatigue. |
|  | CAUTION |
|  | Danger of cutting injuries on edges and sheet metal parts. - Take special care when handling these components, wear cut-resistant gloves if necessary |

(Re-)Commissioning / decommissioning

(Re-)Commissioning

Trained and instructed specialist personnel may only carry out the (re-) commissioning of the system.

Before commissioning the system, the following conditions must be met:

- There are no bare wire ends or they have been secured against accidental contact.

- Checks and signal tests must be carried out for the complete system; in the case of modifications/conversions, checks and signal tests must be carried out for the modified part of the system.

- Order and cleanliness must be ensured after completion of the work.

- It must be ensured that no persons are present in the danger zone.

|  | DANGER |
| --- | --- |
|  | Electrical voltage A qualified electrician may only make all electrical connections to a properly installed power supply system in compliance with the applicable safety regulations for work on electrical system parts. |

Decommissioning

The decommissioning of the electrical system includes switching off before all inspection, maintenance and repair work, as well as before a shutdown period.

- Switching off the mains disconnection device or a maintenance switch.

- Secure against reconnection.

- If necessary, depressurize other media (e.g. water, compressed air, hydraulics) and disconnect them from the media supply.

|  | DANGER |
| --- | --- |
|  | Remaining risk! Excluded circuits and residual voltages! When the mains disconnecting device is switched off, specially marked external circuits (e.g. for lighting, sockets, ...) may still be live (excluded circuits)! Some equipment (servocontrollers, frequency converters, ...) with an electrical DC link may still have dangerous residual voltages for a certain time after disconnection. Check that no voltage is present before starting work. |

Cleaning

The control cabinet should be cleaned dry or damp at least every six months, whereby the cleaning interval must be adjusted depending on the degree of contamination.

Dry cleaning

- Disconnect the control cabinet from the power supply before cleaning.

- Clean the outer surfaces of the control cabinet with a dry, clean cloth.

|  | DANGER |
| --- | --- |
|  | Risk of death or injury from electric shock on contact with live parts. Do not open the housing during cleaning. |

Damp cleaning

If the control cabinet is heavily soiled, damp cleaning of the outer surfaces is also possible. Regular cleaning ensures trouble-free operation and safety.

- Only use cleaning agents that are solvent-free, water-soluble and non-flammable.

- Do not use high pressure or aggressive cleaning equipment.

- Dispose of used cleaning agents and auxiliary materials properly and correctly.

- Observe all cleaning instructions in the documentation of the sub-suppliers.

- Disconnect the control cabinet from the power supply before cleaning.

- Clean only the outer surfaces of the control cabinet with clean water, the approved cleaning agents and a clean, damp cloth.

|  | DANGER |
| --- | --- |
|  | Risk of death or injury from electric shock on contact with live parts. - Before starting wet cleaning, have a qualified electrician disconnect the control cabinet from the main supply line. - Do not open the housing during cleaning. - Ensure that no water gets to live parts. - Do not use high-pressure cleaners. |
|  | CAUTION |
|  | Threat of property damage due to water penetration and incorrect cleaning agents. - Do not open the housing during cleaning. - Ensure that no water gets to live parts. - Do not use high-pressure cleaners. - Ensure that only approved cleaning agents are used. |

Maintenance / repair

|  | NOTE |
| --- | --- |
|  | Operating personnel in accordance with chapter 3.3.4 „Personnel selection“ may only carry out maintenance, repair and overhaul of the electrical system. Information on maintenance in the documentation of the sub-suppliers must be observed. |

The operator must have the electrical system checked at regular intervals by a qualified electrician to ensure that it is in proper condition. The regular maintenance intervals must be adapted to the operating conditions and locations.

|  | DANGER |
| --- | --- |
|  | Electrical voltage - Work on electrical equipment may only be carried out by a qualified electrician. - The five safety rules must be observed; these are assumed to be known by a qualified electrician. - If disconnecting devices are switched off in order to work on electrical or moving components, the disconnecting device must be secured in the OFF position to prevent it from being switched on again (e.g. by padlocks; warning sign if disconnecting device is inside housing). |
|  | DANGER |
|  | Remaining risk! Excluded circuits and residual voltages! When the mains disconnecting device is switched off, specially marked external circuits (e.g. for lighting, sockets, ...) may still be live (excluded circuits)! Some equipment (servocontrollers, frequency converters, ...) with an electrical DC link may still have dangerous residual voltages for a certain time after disconnection. Check that no voltage is present before starting work. |

|  | NOTE |
| --- | --- |
|  | Safety-related components must be replaced at the end of their service life in accordance with EN ISO 13849-1. (See 3.5: "Protective devices") |

Maintenance table

| Pos. | Device | Activity | Description | D | M | Y |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Control cabinet and terminal box | Check for any external damages. | In case of damage, contact a qualified electrician. Do not use the control cabinet any longer. |  | 6M |  |
| 2 | Control cabinet and terminal box | Check for unused openings and missing cable glands | Seal unused openings with dummy plugs. |  | 6M |  |
| 3 | Ventilation systems | Check filter mats for contamination - Clean if dirty - If too dirty, replace according to manufacturer's instructions |  |  | 6M |  |
| 4 | Air conditioning units | Check filter mats for contamination - Clean if dirty - If too dirty, replace according to manufacturer's instructions |  |  | 6M |  |
| 5 | Bolted joints | Check all screw connections and terminals for tight fit | Tighten loose screws. |  |  | 1Y |
| 6 | Warning signs | Warning signs must be clearly visible, not covered and not damaged. | Replace damaged warning signs, remove covering material |  |  | 1Y |
| 7 | Electrical installation | Carry out tests according to IEC 60364-6 | Execution by a qualified electrician only |  |  | 4Y |
| 8 | Electrical installation | Ground fault circuit interrupter in stationary systems | Check for proper function by actuating the test device |  | 6M |  |
| 9 | Electrical installation | Check cables and connections, plugs, devices in the control cabinets, for damage and tight fit | Immediately replace damaged cables, connections, plugs/sockets, protective devices, equipment, etc. with identical or similar components with the same characteristic values. |  | 6M |  |

Repair

After repairs, the following must be observed:

- Adjustable protective devices must be set according to the values as specified in the operating instructions/circuit diagram.

- If necessary, the rotating field must be checked after replacing components.

Disassembly

The generally applicable rules of technology, taking into account the accident prevention regulations and the harmonized standards for occupational safety, must be observed.

Observe the disassembly instructions of the component suppliers in their operating instructions!

For assemblies and components with a weight greater than 15 kg, suitable lifting equipment must be available.

Electrical disassembly

|  | DANGER |
| --- | --- |
|  | Electrical voltage - Work on electrical equipment may only be carried out by a qualified electrician. - The five safety rules must be observed; these are assumed to be known by a qualified electrician. - If disconnecting devices are switched off in order to work on electrical or moving components, the disconnecting device must be secured in the OFF position to prevent it from being switched on again (e.g. by padlocks; warning sign if disconnecting device is inside housing). |
|  | DANGER |
|  | Remaining risk! Excluded circuits and residual voltages! When the mains disconnecting device is switched off, specially marked external circuits (e.g. for lighting, sockets, ...) may still be live (excluded circuits)! Some equipment (servocontrollers, frequency converters, ...) with an electrical DC link may still have dangerous residual voltages for a certain time after disconnection. Check that no voltage is present before starting work. |

Mechanical disassembly

- Observe the disassembly sequence for assemblies, prevent dangerous movements (transport lock, define safe positions, ...)

- Observe the disassembly sequence for assemblies that can lead to a loss of stability.

|  | WARNING |
| --- | --- |
|  | Falling components - Always carry out any disassembly work from top to bottom. - Watch out for falling assemblies and components. - Secure larger assemblies with suitable load handling devices. |
|  | WARNING |
|  | Risk of falling / tripping - Immediately remove and dispose of spilled liquids and dirt residues on the floor. |

Waste Disposal

|  | Dispose of all residual and waste substances in accordance with the applicable state of legislation. Sort the recyclable parts and take them to a proper recycling center. When disposing of the consumables used, observe the safety data sheets (MSDS) and specifications provided by the manufacturers respectively! Consult with the local authorities or a specialist waste disposal company if you require information about environmentally friendly waste disposal. |
| --- | --- |