Layout Conveyor ++ST032

Production process in ++ST032

ST032 is a station where two sets of PLCS operate in coordination. HG01 is solely responsible for transportation. The battery can only enter ST032 when an PNPN signal is sent to “part request”.

When the battery enters TL003(which has an occupied status and type designation and no completion signal),the TK lifts , followed by the TV rising to the working position.

A”in position”PNPN signal isthen sent to acrylic , along with the program number, notifying it to begin work. Upon completion of theoperation,a “part release” signal will be sent, and the battery will be conveyed out.

General Information

|  | NOTE |
| --- | --- |
|  | The light curtain is interrupted when opening the safety door and need to be reset on both sides. |

Constituents of ++ST032

This transmission lines consists of the following components with an operating mode:

- ++ST032+TL001: Roller conveyor

- ++ST032+TL003: Roller conveyor

- ++ST032+TK002:Rotating conveyor

- ++ST032+TK004: Rotating conveyor

- ++ST032+SG001: Pneumatically movable bulkhead

This transmission lines consists of the following components without an operating mode:

- ++ST032+SD001: Protective door

- ++ST032+OP001: Operation Panel (HMI)

Special operating mode functions

Controlled Stop

When the special operating mode “Controlled Stop” is selected on the GROUP Level all components of this group will stop the movement in the next possible step without interrupting a running process.

Stop in Home

When the special operating mode “Stop in Home” is selected on the GROUP Level all components of this group will stop the movement in the next possible Home position.

Dry run

The special operating mode “Dry Run” is not defined and not programmed for this station.

Run empty

When the special operating mode “Run empty Group” is selected all clusters trying to enter the line will be stopped at the conveyor ++ST032+TL001 in front of the laser cabin.

Special functions

General Information

There are no special functions in this station.

Available Types

In this station there are two types available for the type dataplaces.

S_EUSG: Is the Type number 1. It is the smaller cluster of cells

S_CKSB: Is the Type number 2. It is the smaller cluster of cells

S_CKSG: Is the Type number 3. It is the smaller cluster of cells

M_EKSG: Is the Type number 4. It is the larger cluster of cells

MHEKSG: Is the Type number 5. It is the larger cluster of cells

M_EKSB: Is the Type number 6. It is the larger cluster of cells